ORIGINAL INSTRUCTIONS

INSTRUCTION MANUAL

PETROL LOGSPLITTER

FFBT10T100

READ THIS INSTRUCTIONS MANUAL BEFORE USE THE TOOL

FEIDER 32, rue Aristide Bergès - ZI 31270 Cugnaux – France

P1

Warning symbols

|  | Read manual before operation |
| --- | --- |
|  | Use safety footwear |
|  | Use work gloves |
|  | Wear face protection |
|  | Wear a safety helmet |
|  | Use hearing protection |
|  | No trespassing |
|  | No smoking in the working area |
|  | Do not spill hydraulic oil on the floor |
|  | Keep your work space tidy! Untidiness can lead to accidents |
|  | If you use a crane, put the lifting gear around the casing. Never lift the firewood splitter at its handle |
|  | Dispose of waste oil properly (local waste oil collection point). It is prohibited to dump waste oil into the environment |

P2

|  | It is prohibited to remove or modify guard and safety mechanism |
| --- | --- |
|  | Only the operator may stand within the operating range of the machine. Keep non-involved persons, as well as domestic and farm animals, out of the hazard area (minimum distance: 5m) |
|  | Cutting and crushing hazard. Never touch dangerous areas when the splitting knife is moving |
|  | Always pay full attention to the movement of the splitting knife |
|  | Before transport. Disconnect the main plug. |
|  | Before operation open the filling plug two rotations. Before transport close it. |
|  | When transporting, the cylinder must always be brought to the transport position |

P3

- Lifting device; - Chain; - Cylinder fixing part; - Splitting knife; - Side retaining arm; - Gasoline engine; - Operation lever; - U-type return bracket; - Protection cover; - Hydraulic hose; - Cylinder; - Output hydraulic hose; - Hydraulic pump; - Wheel; - Axle; - Fuel tank; - Lgnition switch.

P4

VLS10T-106E  Technical Data

| Technical Data |  |  |
| --- | --- | --- |
| Engine |  | Gasoline engine |
| Engine power | Ps/kw | 4-stroke engine-6.5/4.8 |
| displacement | cc | 196 |
| Motor Starter |  | Recoil Starter (Pull Starter) |
| Fuel |  | Unleaded gasoline octane number from 90 to a maximum ethanol content of 5% |
| Fuel capacity | l | 3 |
| Motor Oil |  | SAE 10W-30 or SAE 10W-40/0.6L |
| Noise LWA | Db(A) | 106 |
| Dimensions L/W/H | mm | 1260*1020*2360 |
| Splitting force | ton | 10 |
| Weight | kg | 179 |
| Working speed | cm/s | 10±0.5 |
| Return speed | cm/s | 7±0.5 |
| Piston stroke | cm | 86 |
| Hydraulic oil capacity | l | 15 |

P5

wiring diagram

P6

Hydraulic diagram

P7

Introduction of parts

Fig1

P8

Fig2

Fig3  Fig4

P9

Fig.5Fig.6

Fig.7Fig.8

P10

Fig9

Safety information

Do not install, repair, clean or handle the log splitter when the machine

is still running and Without having securely clamped the wedge.

Severe personal injury or machine damages can result from not complying with operation, assembly, maintenance and repair instructions contained in this manual.

Allow only adults to operate/maintain the log splitter after they have received sufficient training and have made themselves familiar with the machine.

Make sure that full compliance is assured at all times with the general safety and health rules on the workplace as well as the applicable local traffic rules.

P11

No one under the age of 18 should be allowed to operate the log splitter. However, young people in age of 16 or slightly more may work on the machine providing that they received adequate training, they carry all due personal protection safeties and that an adult supervisor keeps standing nearby.

Machine instability can result in injury or severe damages,

To ensure stability during operation make sure to choose a flat, dry floor free from any tall grass, brush or other interferences.

To avoid tripping, do not leave tools, logs, or other components laying around in the work area. Avoid slippery floor conditions by eventually scattering saw dust or wood pellets.

Following precautions must be taken at all times

-NEVER use your log splitter at night or without sufficient illumination.

-NEVER operate your log splitter on slippery, wet, muddy, or icy surfaces.

-NEVER let electrical works be carried out by unskilled staff.

-NEVER operate the machine without wearing protective shoes, tight-fitting gloves and apparels.

-NEVER remove from your log splitter the safety tools and devices mounted on the

Make sure to select a suitable place with sufficient clearance for proper operation of the machine as well as or safe handling of the split logs and eventual ancillary tools and equipment.

1.Mandatory use and application

The log splitter is strictly designed for one-man operation. Never allow more than oneperson approach and work on the machine at the same time.

This splitter is conceived for splitter short and long logs for firewood preparation only.

BEWARE: no log splitting is permitted.

When placing the log on the log splitting make sure that the chunk rests entirely and safely on the base plate of the machine.

P12

BEWARE: this unit has a log capacity of min 70 mm and max 400 mm diameter.

Danger! Keep clear of moving parts!

2 .APPLICATION CONDITIONS

This log splitter is designed for operating under ambient temperatures between +5℃ and 40℃ and for installation at altitudes no more than 1000m above M.S.L. The surrounding humidity should less than 50% at 40℃.It can be stored or transported under ambient temperatures between -25℃ and 55℃.

3.Never dismantle and reassemble the log splitter by yourself unless you are a professional engineer. Otherwise, the self-assembling may lead to some dangers as below:

(1) Oil leak.

(2) No pressure.

(3) Engine & pump damage.

(4) Pressure is too big for cylinder endurance etc.

Intended use

The machine confirms to the current EC Machinery Directive.

.  The machine may be used only in the “standing mode”,logs may be splited only in an  upright state and along the log direction. The dimensions of the log to be split:

Short logs 30cm

Long logs 100cm

.  Never split the log when it is lying down or across the log direction.

.  The safety work and maintenance instructions of the manufacturer as well as the dimensions indicated in the technical data must be followed.

.  The relevant accident prevention regulations and other generally accepted safety rules must be followed.

.  The machine may be used serviced or repaired only by people who are familiar with it and are instructed about the hazards.

P13

Independent modifications to the machine exclude liability of the manufacturer for resulting damage.

.  The machine may be used only with original accessories and tools of the manufacturer.

.  Any other use is considered improper. The manufacturer does not assume liability for resulting damage; the associated risk is solely borne by the plant operator.

Transport/storage

Before handing, moving or transporting the splitter make sure to cut the power off (unplug the machine).

Transport with forklift trucks/lifting cars:

Before delivery the machine should be secured with steel tapes and securely mounted on a pallet.The machine is protected by a plastic sheet. To lift it off the pallet several persons or technical resources are required.

Note: the center of gravity of the log splitter is high-risk of tripping over!

Transport by crane:

For transport using the hook the crane lifting eye is on the upper part of the machine

Should be used. Never lift on the splitter blade!

At ambient temperatures is below 5℃,the machine should run at idle for about 5 minutes so that the hydraulic system reaches the operating temperature.

Prepare the workplace where you want to install the machine. Create enough space for safe, trouble-free working.

The machine is high and very heavy so while transport must need at least 2 persons.

(see Fig.9)

The machine is designed to work on flat surfaces and must be erected on a level solid base.

P14

Installation

Mount the lifting device Fig.1

With 1 hexagonal bolt M10X55 and one M10 nut (spare parts 3,2) to connect the lifting device (spare part 7),then with 2 hexagonal bolts M10x 25 and two nuts M10 (spare part1,2) to fix the support of the lifting device(spare part 4), and adjust the gap between the liftting device and the support(gap=0.5～1.0).finally connect the chain(spare part 6).

Mount the wheel and axle Fig.2

Place an axle (part 3) on the splitter through the hole of the base palette.

Assemble two wheels (part 2) on both sides of the axle and lock it with two cotter pins (part 3).

For reference according to the following pictures.

P15

P16

Assemble the control handle Fig.3

Because of packing reason you must assemble two control handles yourself. You can see Fig 3, and mount the two handles. (with tool:13-15 wrench)

Part 1---- hexagonal nut M8,

Part2----handle

Part3---stop collar

Part4---R pin.

Because of packing reason you must mount two control handles yourself. You can see it on the mounted control handle and at picture Fig.3  According to the following steps:

P17

Mount the Side retaining arm Fig.6

Mount the two side retaining arms with 3 hexagonal bolts M10*45+1 special bolt (part 2) and 4 nuts M10(part 3) (with tool:17-19 wrench).

another useful of the special bolt (part 2) is connecting the fixing part of the cylinder and secure it with a R-pin(part 4).

For reference, can see the following pictures.

P18

Working notes

Bring the log splitter to working position. Fig.4

Because of packing reasons the cylinder was descended in the lowest position so after the engine is running the cylinder must raise to working position

Press two operation handles downward until the cylinder raise to the top of the machine, then insert two shafts (2)into the holes of the machine and lock with two R-pins (1) to secure the cylinder on the machine.

For reference please according to the following steps:

P19

Caution!

Before delivery, while packing or transport，the cylinder must be descended from top position to the lowest position and fixed in the fixing part of the cylinder (Fig.9 cylinder fixing part).

According to the followings:

Press the two operation handles downwards the riving knife begins to work forward until to the cylinder fixing part, release one operation handle, pull out two R-pins and two shafts, then operate the two operation handles upward , the cylinder begin to descend, till to the properly position release two operation handles, stop the machine and put two shafts、two R-pins into the Accessories bag. Finally, remove the cylinder fixing part and insert it into long bolt then lock it with R-pin.

Start-up

Before the first use, make sure that the log splitter is in good conditions and that no visual damages are there.

Check all hydraulic hoses, fittings and coupling and to detect and repair eventual oil leaks.

Make sure that all safeties and protections are duly assembled on the machine. Do not attempt to remove or by pass these safeties!

Make sure the that the log splitter will not be damaged or made unsafe by any operation, lubrication, maintenance, or repair procedures that you choose. Should any trouble or unusual behavior be detected, do not start splitting wood until these have been fixed.

Before working the machine, please release the bleed screw so that the air can escape from the oil tank.

Leave the bleed screw open during the operation.

Before you move the machine, close the bleed screw again in order not to lose any oil.

Refueling, see p4. part 16(fuel tank)

Fill only with clean fuel with a minimum of 90 octane and a maximum of 5% of bio-ethanol in the fuel tank (p4. part 16). With the help of the fuel gauge, you can check the level of fuel in the tank comfortably even during operation.

P20

Start the engine, see the following picture.

Follow the procedures below for a cold start:

- Open the fuel valve(B) to the right in the opening position.

- Turn the choke lever (A) on the engine to fully activated position to the left.

- Move the throttle lever(C)on a semi-open position.

- Turn the engine switch (E)to ON.

- Pull slowly several times the reverse starter handle(D) so that the fuel flows into the

carburator. Then hold the reverse starter handle(D) and pull the cable out slightly until you feel resistance. Then pull the cable in a quick motion and let the cable rewind slowly again. Do not allow the cable to snap back. Tighten if necessary, the reverse starter handle(D) repeatedly until the engine starts.

- Let the engine warm up for a few seconds. Then gradually move the choke lever(A)to the right in the “OPEN” position.

The restart of an engine that is already hot from the previous operation usually does not require use of the choke.

- Open the fuel valve(B) to the right in the opening position.

- Move the throttle lever(C)on a semi-open position.

- Turn the engine switch (E)to ON.

- Then hold the reverse starter handle(D) and pull the cable out slightly until you feel resistance. Then pull the reverse starter handle(D) in one swift movement and leave   the cable to wrap back slowly .Do not allow the reverse starter handle(D) to snap back.

P21

After the engine starts, move the throttle lever(C)to full throttle position, so that the splitter can achieve its maximum working force.

End of work

. Move the riving knife to the lower position

. Release the operation lever;

. Turn off the machine by turning the ignition switch(E) to off.

. Close the fuel valve(B)

. Close the bleed screw

Functional test

Before each use, perform a function test.

| action | result |
| --- | --- |
| Press both handles downwards | The riving knife goes down till to 18cm above the base plate. |
| Press return bracket or both handles upwards | The riving knife return to selected upper position. |
| Release both handles | The riving knife remains in the selected position. |

Bring the log splitter to working position. Fig.4

Because of packing reasons the cylinder was descended in the lowest position, so after the engine is run, the cylinder must raise to working position.

Press two operation handles downward until the cylinder raise to the top of the machine, then insert two shafts(2)into the holes of the machine, and lock with two R-pins (1) to secure the cylinder on the machine.

P22

Caution!

Before delivery, while packing，the cylinder must be descended from top position to the lowest position, and fixed in the fixing part of the cylinder (Fig.8 cylinder fixing part).

According to the followings:

Press the two operation handles downward, the riving knife begins to work forward, until to the cylinder fixing part, release one operation handle, pull out two R-pins and two shafts ( see Fig.4 ),then operate the two operation handles upward , the cylinder begin to descend, till to the properly position, release two operation handles, stop the machine, and put two shafts、two R-pins into the Accessories bag .

Maintenance instructions

Do not install, repair, clean or handle the log splitter when the machine is still

running and without having securely clamped the wedge.

Daily maintenance

Make maintenance regular part of daily operation. The daily maintenance routine needs to include

- Cleaning of the machine and cleaning of all parts from residual wood debris, chips, dust, bark pieces and eventual other waster.

- Hydraulic oil check and (in case of leakage) hydraulic hose and fittings check-up to detect eventual oil leaks

- Lubrication of all moving parts

Hydraulic oil

Periodically check the oil level inside the hydraulic oil tank. When doing so, accurately avoid contaminating the tank with dirt, wood chips, sow dust etc…

Make sure that the splitter never runs without oil or with low oil level. When this

happens, air is likely to reach inside the hydraulic loop. Failure to maintain due oil or level may cause poor running and irregular splitter operation (very rough, back/forth or up/down motions) as well as major pump damages.

P23

Please schedule your first oil change after approximately 25-30 operation hours and later ones after each 50operation hours or once a year.

The oil drain plug is location on the bottom tank side while the filler cap is on the top of the machine (see Fig .5).

The hydraulic system is a closed system with oil tank, oil pump and control

valve. Check oil level regularly in inspection glass. Low oil levels can damage the oil pump. Oil Level should measure approx.1—2cm lower than the lower surface of the oil tank.

The oil should be completely changed once a year.

- Make sure moving parts stops and the log splitter is unplugged.

- Make sure that no dirt or debris finds its way into the oil tank.

- Collect used oil and responsibly recycle.

- Following an oil change, activate the log splitter a few times without actually splitting

Recommended oil type: DEA HD B46, Shell Tellus 10-46, Esso Nuto H 46

When changing the oil, never let used oil drop down on the ground, rather collect whole of it in a sealed container for due disposal. Oil disposal container should be of at least 15liter capacity if you are using smaller container make sure to drain the more than one round to avoid spilling old oil out on the ground.

Used oil is very polluting and should.

Do not install, repair, clean or handle the log splitter when the machine is still running and without having securely clamped the wedge.

Treat the disposal matters in accordance with local rules.

After refilling the tank with new oil, let the splitter cycle three or four times and let the air blow out of the hydraulic loop before closing the bleed screw.

Make a trial test before starting to work again. The noisy knock should disappear after greasing.

Should you detect an unusually large gap between the riving knife and the guide, sliding pads are likely to be worn out, if so replace them with brand new ones.

P24

Risk factors

The special two –hands control mechanisms minimizes risks and dangers related to moving parts on the machine.

An additional safety provided on the machine in order to prevent working with only one handle while the other one is being engaged (held down) by some mechanical tool or system.

DO NOTEVERATTEMPT to remove or by-pass the two-hand control

Danger: operating the splitter without the safety two-hand control will increase your risk of having your hands pinched during the splitting cycle.

Do not remove any other safety and protection device the machine.

WARNING: splitting without due safety devices might result into serious injures to the operator or the other person around the workplace.

Keep hands and fingers clear at all time. Many log-splitting accidents happen on the return portion of the stroke.

P25

Trouble shooting chart

| Type of malfunction | Possible origins of the problem | Solutions |
| --- | --- | --- |
| When the control hands is pushed down. The wedge doesn’t tower | -Oil level is too low | -Fill oil in |
| Wedge moves slowly or Will not extend completely on normal log. | -Oil level is too low -Wrong valve détente -Pump is broken -Cylinder gaskets are damaged | -Fill oil in -Adjust valve -Replace pump -Replace gaskets |
| Ram stops during splitting | -Log is too big, crogged or with lots of branches | -Turn/adjust log position |
| Hydraulic hoses heat-up excessively | -oil level too low -Pump is damaged -Wrong valve | -Fill oil in -Replace pump -Adjust valve |

EG-Declaration of conformity

FEIDER
   32 rue Aristide Berges -Z1 31270 Cugnaux - France
Tel: +33 (0) 5.34.508.508 Fax: +33 (0) 5.34.508.509

Designation of the machine:

Petrol Logsplitter 3000W

Model:

FFBT10T100

We hereby declare that the machine described below due to its design and construction as well as the model marketed by us, comes with the relevant regulation of the EC Directives listed below.

In case of modifications to the machine, the present declaration loses its validity.

Relevant EC directives:

EC machine directive 2006/42/EG

EC -EMV directive 2004/108/EWG

ROHS 2011/65/EU

PAHS  AFPS-GS-2014-01-PAK

Standard references:

EN14982

BS EN609-1-2017

Cugnaux, 24/06/2019

Philippe MARIE / PDG